Electric Gear Pump - 351 FA/M075 - 200940

Technical data

- Direction of rotation: right (cw), view of the pump shaft

- Swallowing volume: 20.92 cm³/rev

- Connections: DIN ISO 228, G 3/4"

- Permissible. Pump inlet pressure: -0.5 ... 0.5 bar

- Operating Temperature Range: -30 to 80°C

- Painted: RAL 7031

Construction materials

- Housing material: EN GJL 250

- Gear material: EN 11SMn30

- Shafts: EN 11SMn30

- Plain bearings: EN GJL 250

- stat. Seals: NBR

- Shaft seal:

  - Radial shaft seal

  - NBR

Three-phase motor

- Motor Size: IEC80

- Engine Beard: IMB34

- Flange diameter: 160mm

- Rated power: 0.75 kW

- Rated speed: 1,425 1/min

- Voltage: Δ 230/Υ 400V, 50Hz, 3~

- Degree of protection: IP 55

- Efficiency class: IE3

- Terminal box: top

Important notes

- Please install a coarse filter with a mesh size of 50 μm in front of the pump

- Pump not suitable for particulate media

- The purity class of the medium should be at least 19/17/14 according to ISO 4406

- Sufficient calming sections in accordance with fluid technology standards are absolutely necessary