Electric Gear Pump - 76 FAL/M025 - 201362

Please note

- Pump not suitable for particulate media

- The purity class should be at least 19/17/14 according to ISO 4406

- Please install a coarse filter with 50μm mesh size in front of the pump

Technical data

- Direction of rotation: left (ccw), view of the pump shaft

- Swallowing volume: 2.8 cm³/rev

- Connections: DIN ISO 228, G 1/4"

- Permissible. Inlet pressure: -0.7...0.5 bar

- Permissible. Differential pressure: 26.3 bar (viscosity dependent)

- Operating temperature range: -20...80°C

- Painted: RAL 7031

Construction materials

- Housing material: EN GJL 250

- Gear material: EN 11SMn30

- Shaft material: EN 11SMn30

- Plain bearing material: EN GJL 250

- stat. Sealing material: NBR

- Shaft seal: radial shaft seal, NBR

Pump drive

- Motor type: three-phase motor

- Standard: IEC 72-1

- Engine size: 71

- Engine design: IMB34

- Flange diameter: ∅140 mm

- Nominal power: 0.25 kW

- Rated Speed: 1425 1/min

- Voltage: 230/400V, 50 Hz

- Degree of protection: IP 55

- Energy efficiency class: ≥ IE1

- Terminal box layer: top