Electric Gear Pump - 5030-350-B-ZK-100-160 - 203038

Technical data

- Explosion Protection: 2014/34/EU - II 2GD cX

- Preferred direction of rotation: left (ccw), view of the pump shaft

- Swallowing volume: 35 cm³/rev

- Safety valve: 0 - 12 bar

- Connections: DIN ISO 228, G 1 1/2"

- Permissible. Pump inlet pressure: -0.9 ... 49 bar

- Operating Temperature Range: -20 to 130°C

Construction materials

- Housing Material: 1.4404

- Gear material: PEEK mod.

- Waves: 1.4571

- Plain bearings: PEEK mod.

- stat. Seals: PTFE

- Shaft seal:

  - max.14 Nm

  - Magnets Sm2Co17, encapsulated in 1.4404

for mounting on a standard engine

- with motor-side magnetic clutch bell

- Motor standard: IEC 72-1

- Size: IEC100

- Type: IMB34

- Flange diameter: 160 mm

- Shaft: 28x60 mm

- Recommended power: 1.5 kW

- Recommended speed: 1,000 1/min

Important notes

- Please install a coarse filter with a mesh size of 50 μm in front of the pump

- Pump not suitable for liquids with magnetic particles

- Pump not suitable for particulate media

- The purity class of the medium should be at least 19/17/14 according to ISO 4406

- Sufficient calming sections in accordance with fluid technology standards are absolutely necessary