Electric Gear Pump - 3030-110-GM-19 - 206090

Technical data

- Preferred direction of rotation: left (ccw), view of the pump shaft

- Explosion Protection: 2014/34/EU - II 2G Ex h IIC Gb X // II 2D Ex h IIIC Db X

- Swallowing volume: 1 cm³/rev

- Connections: DIN ISO 228, G 1/4"

- Permissible. Pump inlet pressure: -0.9 ... 89 bar

- Operating Temperature Range: -20 to 130°C

Construction materials

- Housing Material: 1.4404

- Gear material: PEEK mod.

- Waves: 1.4571

- Plain bearings: PEEK mod.

- stat. Seals: EPDM

- Shaft seal:

  - max.0.65 Nm

  - Magnets Sm2Co17, encapsulated in 1.4404

with electrically commutated external rotor motor Papst/Siemens VDC 3-43.610

- Integrated control electronics

- Rated power: 18.8 W

- Rated speed: 4,000 1/min

- Voltage: DC 24V

- Degree of protection: IP 00

- incl. mounting bracket

Important notes

- Please install a coarse filter with a mesh size of 50 μm in front of the pump

- Pump not suitable for liquids with magnetic particles

- Pump not suitable for particulate media

- The purity class of the medium should be at least 19/17/14 according to ISO 4406

- Sufficient calming sections in accordance with fluid technology standards are absolutely necessary