Gear Flange Pump - 151 FBR/GN/FGG – 201267-A

Please note

- Pump not suitable for particulate media

- The purity class should be at least 19/17/14 according to ISO 4406

- Please install a coarse filter with 50μm mesh size in front of the pump

Technical data

- Direction of rotation: right (cw), view of the pump shaft

- Swallowing volume: 5.71 cm³/rev

- Safety valve: 8-15 bar

- Connections: DIN ISO 228, G 3/8"

- Permissible. Inlet pressure: -0.7...0.5 bar

- Permissible. Differential pressure: 8 bar (viscosity dependent)

- Operating temperature range: -20...160°C

Construction materials

- Housing material: EN GJL 250

- Gear material: EN 11SMn30, ground

- Shaft material: EN 11SMn30

- Plain bearing material: EN GJL 250

- stat. Sealing material: FKM

- Shaft seal: radial shaft seal, 2xFKM

Flange Pump

- with centering ring made of gray cast iron and 4 screw-on holes

- with free shaft end

- Paintwork: RAL 7031

- Valve set to 10 bar