Gear Pump - 451 BRLS/SK14/M1.5 - 201031

Technical data

- Direction of rotation

  - Preferred direction of rotation on the right (cw), view of the pump shaft

  - Left (ccw) without pressure relief valve function

- Swallowing volume: 31 cm³/rev

- Safety valve: 5 - 10 bar

- Connections: DIN 228, G1", screwed-in weld-on nipples

- Permissible. Pump inlet pressure: -0.7 ... 10 bar

- Operating Temperature Range

  - Permanent -15 to 160 °C

  - short-term up to 300°C

Construction materials

- Housing material: EN GJS 400

- Gear material: EN 11SMn30

- Shafts: EN 11SMn30

- Plain bearings: multi-layer plain bearings

- stat. Seals: FKM

- Shaft seal:

  - Stuffing box packing

  - PTFE/Graphite

Foot pump on base plate with motor

- mounted on cast iron base plate

- including clutch and clutch protection

- Attached to a standard engine

  - Motor Size: IEC90L

  - Engine Beard: IMB3

  - Nominal power: 1.5 kW

  - Rated speed: 1,425 1/min

  - Voltage: 230/400V, 50Hz, 3~

  - Degree of protection: IP 55

  - Efficiency class: IE3

  - Terminal box: top

Important notes

- Please install a coarse filter with a mesh size of 50 μm in front of the pump

- Sufficient calming sections in accordance with fluid technology standards are absolutely necessary