Pump Head MV0662 – 4030-710-B-PK-80-A36-1 - 204679

Important notes

- Please install a coarse filter with a mesh size of 50 μm in front of the pump

- Pump not suitable for liquids with magnetic particles

- Pump not suitable for particulate media

- The purity class should be at least 19/17/14 according to ISO 4406

- Sufficient calming sections in accordance with fluid technology standards are absolutely necessary

Technical data

- Explosion Protection: 2014/34/EU - II 2GD cX

- Preferred direction of rotation: left (ccw), view of the pump shaft

- Swallowing volume: 7.1 cm³/rev

- Safety valve: 8 - 15 bar, *** Set to 12 bar ***

- Connections: DIN ISO 228, G 3/4"

- Permissible. Pump inlet pressure: -0.9 ... 88 bar

- Operating Temperature Range: -20 to 130°C

Construction materials

- Housing Material: 1.4404

- Gear material: PEEK mod.

- Waves: 1.4571

- Plain bearings: PEEK mod.

- stat. Seals: PTFE

- Shaft seal:

  - max.3.8 Nm

  - Magnets Sm2Co17, encapsulated in 1.4404

Pump head

- with containment shell and pump-side magnetic clutch hub

- including centering ring, with screw-on holes