COMPETITOR ANALYSIS

Gear Flange Pump 151 FBR/GN/FGG

Scherzinger Model 201267-A

5.71 cc/rev | Cast Iron | Flange Mount | 160°C | FKM Seals | Safety Valve

Prepared for Scherzinger GmbH

Savsen Technologies

April 2026

# 1. Reference Product: Scherzinger 201267-A

The Scherzinger Gear Flange Pump 151 FBR/GN/FGG (Model 201267-A) is a compact, flange-mounted gear pump head designed for lubrication and fluid transfer applications. Key differentiators include its cast iron construction, high temperature range (160°C), FKM (Viton) seals, integrated safety valve, and free shaft end for flexible drive coupling.

| Parameter | Scherzinger 201267-A Specification |
| --- | --- |
| Pump Type | External gear pump (flange mount) |
| Displacement | 5.71 cm³/rev |
| Safety Valve | 8-15 bar (set to 10 bar) |
| Connections | DIN ISO 228, G 3/8" |
| Inlet Pressure | -0.7 to 0.5 bar |
| Max Differential Pressure | 8 bar (viscosity dependent) |
| Temperature Range | -20 to 160°C |
| Housing Material | EN GJL 250 (grey cast iron) |
| Gear Material | EN 11SMn30, ground |
| Shaft Material | EN 11SMn30 |
| Plain Bearings | EN GJL 250 |
| Static Seals | FKM (Viton) |
| Shaft Seal | Radial shaft seal, 2x FKM |
| Mounting | Flange with centering ring, 4 bolt holes |
| Shaft | Free shaft end (no integrated motor) |
| Paint | RAL 7031 (blue-grey) |
| Filter Requirement | 50 μm coarse filter upstream |
| Media Purity | ISO 4406 class 19/17/14 minimum |

## Key Differentiators vs. Typical Competitors

- 160°C temperature range: Most competitors are limited to 80-100°C with standard NBR seals. The FKM (Viton) seals on the 201267-A enable operation at temperatures far beyond what standard pumps can handle.

- Integrated safety valve (8-15 bar, set to 10 bar): Very few small gear pumps include a built-in safety valve. Most require external relief valves, adding cost and complexity.

- Cast iron construction (EN GJL 250): While many Group 1 competitors use aluminum bodies, the Scherzinger uses cast iron throughout (housing, bearings), providing superior durability and thermal stability.

- Dual FKM shaft seals: Double-sealed design with Viton for enhanced reliability at high temperatures. Standard competitors typically use single NBR seals.

- Flange mount with centering ring: Precision centering ring ensures accurate alignment — critical for longevity in high-temperature lubrication systems.

# 2. Competitor Overview: 12 Comparable Products

The following table summarizes 12 competitor products from 8 countries. Note that very few competitors offer the combination of cast iron housing, integrated safety valve, FKM seals, and 160°C temperature range that the Scherzinger 201267-A provides.

| # | Manufacturer | Model | Origin | Displ. | Pressure | Material | Temp | Pump Price |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Marzocchi | ALP1A-D-6 | Italy | 4.1cc | 250 bar | Alum | ~90°C | USD 196 |
| 2 | Casappa | PLP10.5D0 | Italy | 5.34cc | 250 bar | Alum/CI | ~100°C | USD 200-350 |
| 3 | Bosch Rexroth | AZPF-11-005 | Germany | 5cc | 250 bar | CI/Alum | ~120°C | EUR 250-500 |
| 4 | Parker | PGP505A-0060 | USA | 6cc | 275 bar | Aluminum | ~100°C | USD 300-500 |
| 5 | KRACHT | KF6RF1-D15 | Germany | 6cc | 250 bar | Cast Iron | ~150°C | EUR 400-650 |
| 6 | Danfoss | SNP1/6.0D | Italy/DK | 6cc | 230 bar | Alum/CI | ~100°C | EUR 100-250 |
| 7 | Bucher | AP100/5.0D880 | Switzerland | 5cc | 250 bar | Cast Iron | ~120°C | EUR 180-400 |
| 8 | Honor | 2GG1U06R | Taiwan | 6cc | 241 bar | Aluminum | ~90°C | USD 245 |
| 9 | CBN/Chinese | CBN-E306 | China | 6cc | 160 bar | Aluminum | ~80°C | USD 20-60 |
| 10 | Settima | GR20 SMT8B | Italy | 6.4cc | 25-40 bar | CI/Alum | ~100°C | EUR 200-500 |
| 11 | Vivoil | XP101 | Italy | ~6cc | 250 bar | Aluminum | ~90°C | EUR 80-200 |
| 12 | Indian OEM | FTMP-6 | India | 6cc | 100 bar | Cast Iron | ~80°C | USD 36-95 |

Note: Prices are estimates based on online listings and industry data as of Q1 2026. 'CI' = Cast Iron. Temperature ratings are approximate maximums with standard seals; FKM upgrade extends range on some models. Very few competitors match the Scherzinger's 160°C.

# 3. Detailed Competitor Profiles

Each competitor below includes full specifications, pricing, source links, and comparison notes against the Scherzinger 201267-A.

## 1. Marzocchi Pompe ALP1A-D-6 (Italy)

| Manufacturer | Marzocchi Pompe |
| --- | --- |
| Model | ALP1A-D-6 |
| Country of Origin | Italy |
| Displacement | 4.1 cc/rev |
| Max Pressure | 250 bar (3625 psi) |
| Max Speed | 4000 rpm max |
| Construction Material | Aluminum body, steel gears |
| Pump Price (Est.) | USD 196 (OneHydraulics, sale) USD 213 (regular) |
| Price Notes | EUR 80-140 (EU distributors est.) |
| Geographic Availability | Italy (manufacturer), USA (OneHydraulics, USC Hydraulics, Amazon), UK (JBJ Techniques), Spain (Hidraoil) |

Source 1: OneHydraulics (USA)

Source 2: USC Hydraulics (USA)

Analysis: Italian Group 1 pump. Closest standard size is 4.1cc (ALP1A-D-6) or 7.6cc (ALP1A-D-11). SAE AA 2-bolt mount. Aluminum construction is lighter but less durable than Scherzinger's cast iron. No integrated safety valve — must be added externally. Max temp typically 90°C vs Scherzinger's 160°C.

## 2. Casappa PLP10.5D0 (Italy)

| Manufacturer | Casappa |
| --- | --- |
| Model | PLP10.5D0 |
| Country of Origin | Italy |
| Displacement | 5.34 cc/rev |
| Max Pressure | 250 bar (3625 psi) |
| Max Speed | 4000 rpm rated |
| Construction Material | Aluminum body, cast iron flange option |
| Pump Price (Est.) | USD 200-350 (US distributors) EUR 150-280 (EU estimated) |
| Price Notes | Higher for cast iron flange variants |
| Geographic Availability | Italy (manufacturer), USA (OneHydraulics, Fremont), Spain (Hidraoil), UK (VHS Hydraulics), India |

Source 1: OneHydraulics (USA)

Source 2: Hidraoil (Spain)

Analysis: Italian manufacturer. PLP10 series Group 1 pump. Available sizes: 3.34, 5.34, 6.67, 8.51, 10.67 cc. Cast iron flange and cover option available. Very close displacement match at 5.34cc. No integrated safety valve. Standard seals limit temp to ~100°C; FKM available on request.

## 3. Bosch Rexroth AZPF-11-005 / AZPB-22-005 (Germany)

| Manufacturer | Bosch Rexroth |
| --- | --- |
| Model | AZPF-11-005 / AZPB-22-005 |
| Country of Origin | Germany |
| Displacement | 5 cc/rev |
| Max Pressure | 250 bar (3625 psi) |
| Max Speed | 3500 rpm |
| Construction Material | Cast iron or aluminum housing |
| Pump Price (Est.) | EUR 250-500 (EU distributors) USD 350-650 (US market) INR 35,610+ (India, IndiaMART) |
| Price Notes | Premium German pricing |
| Geographic Availability | Germany (manufacturer), UK (Hydraulics Online), India (IndiaMART), Poland (Machineryline), USA (USC Hydraulics), Mexico |

Source 1: Hydraulics Online (UK)

Source 2: IndiaMART (India)

Analysis: German premium manufacturer. AZPF Series 1X offers 4 and 5cc sizes. AZPB series for smaller platform. Cast iron housing matches Scherzinger 201267-A. Available in Standard, High-Performance, SILENCE variants. No integrated safety valve in standard config. FKM seals available as option.

## 4. Parker Hannifin PGP505A-0060 (USA)

| Manufacturer | Parker Hannifin |
| --- | --- |
| Model | PGP505A-0060 |
| Country of Origin | USA |
| Displacement | 6 cc/rev |
| Max Pressure | 275 bar (3988 psi) |
| Max Speed | 4000 rpm max |
| Construction Material | Aluminum body, bushing block |
| Pump Price (Est.) | USD 300-500 (US distributors) |
| Price Notes | Highest pressure rating in comparison |
| Geographic Availability | USA (manufacturer), UK (Hydraulics Online), Spain (Hidraoil), worldwide distribution |

Source 1: Parker Official (USA)

Source 2: GS Global Resources

Analysis: PGP505 Group 1 series. Sizes 2-13cc. CW, CCW, and bi-rotational options. Extensive valve options including pressure relief, anti-cavitation, cross port relief, solenoid unloading. Highest pressure rating at 275 bar. Aluminum construction. Good North American availability.

## 5. KRACHT GmbH KF6RF1-D15 (Germany)

| Manufacturer | KRACHT GmbH |
| --- | --- |
| Model | KF6RF1-D15 |
| Country of Origin | Germany |
| Displacement | 6 cc/rev |
| Max Pressure | 250 bar |
| Max Speed | 3000 rpm |
| Construction Material | Cast iron housing, hardened steel gears |
| Pump Price (Est.) | EUR 400-650 (EU estimated) GBP 350-550 (UK estimated) |
| Price Notes | Premium German cast iron construction |
| Geographic Availability | Germany (manufacturer), UK (White House Products), USA (KRACHT Corp), China (Alibaba resellers) |

Source 1: KRACHT Media (catalog)

Source 2: KRACHT Corp (USA)

Analysis: German manufacturer from Werdohl. KF3-KF6 range covers small displacement. CAST IRON housing — direct match to Scherzinger 201267-A material. DIN flange mounting available (F/G variants). Closest competitor in construction quality and material. Available with outboard bearing option.

## 6. Danfoss (Turolla) SNP1/6.0D (or SKP1/6.0D) (Italy / Denmark)

| Manufacturer | Danfoss (Turolla) |
| --- | --- |
| Model | SNP1/6.0D (or SKP1/6.0D) |
| Country of Origin | Italy / Denmark |
| Displacement | 6 cc/rev |
| Max Pressure | 230 bar |
| Max Speed | 3500 rpm |
| Construction Material | Aluminum/cast iron options |
| Pump Price (Est.) | EUR 100-250 (EU distributors) USD 150-350 (US market) |
| Price Notes | Good price-performance ratio |
| Geographic Availability | Italy (Turolla HQ), Denmark (Danfoss HQ), Spain (Hidraoil), UK (White House Products, Hydraulics Online), USA (Legacy Hydraulics) |

Source 1: Hidraoil (Spain)

Source 2: Hydraulics Online (UK)

Analysis: Turolla (now Dana/Danfoss) Group 1 series. SNP1 and SKP1 variants. 11 displacement sizes including 6.0cc. Tapered shaft options. Widely used in mobile hydraulics and agricultural equipment. Lower pressure rating than some competitors but excellent price-performance.

## 7. Bucher Hydraulics AP100/5.0D880 (Switzerland)

| Manufacturer | Bucher Hydraulics |
| --- | --- |
| Model | AP100/5.0D880 |
| Country of Origin | Switzerland |
| Displacement | 5.0 cc/rev |
| Max Pressure | 250 bar |
| Max Speed | 3000 rpm |
| Construction Material | Cast iron housing (D880 variant) |
| Pump Price (Est.) | GBP 116-139 (Flowfit UK, smaller sizes) USD 200-450 (US estimated) EUR 180-400 (EU estimated) |
| Price Notes | Cast iron D880 variant is premium |
| Geographic Availability | Switzerland (manufacturer), UK (Flowfit), USA (GS Global Resources, Amazon, Walmart), Germany |

Source 1: Bucher Official

Source 2: Flowfit (UK)

Analysis: Swiss manufacturer. AP100 series for small displacements. Available in both aluminum (S309) and CAST IRON (D880) housing — D880 matches Scherzinger material. Displacements: 1.2, 2.5, 4.3, 5, 6.5cc. Compact design. DIN flange mounting. Can be combined into double/triple pump assemblies.

## 8. Honor (Taiwan) 2GG1U06R (Taiwan)

| Manufacturer | Honor (Taiwan) |
| --- | --- |
| Model | 2GG1U06R |
| Country of Origin | Taiwan |
| Displacement | 6 cc/rev (0.37 in³) |
| Max Pressure | 241 bar (3500 psi) |
| Max Speed | 4000 rpm |
| Construction Material | Aluminum body |
| Pump Price (Est.) | USD 245.27 (OneHydraulics, sale) USD 354.28 (regular) |
| Price Notes | Mid-range pricing with good specs |
| Geographic Availability | Taiwan (manufacturer), USA (OneHydraulics), global distribution |

Source 1: OneHydraulics (USA)

Source 2: OneHydraulics - Honor Collection

Analysis: Taiwanese manufacturer. Group 2 pump body with 6cc displacement. SAE A 2-bolt mount. Multiple shaft options (keyed, splined). Available in CW and CCW rotation. Aluminum construction. Good quality for the price point. Well-stocked at US distributors.

## 9. CBN/Chinese Generic CBN-E306 (China)

| Manufacturer | CBN/Chinese Generic |
| --- | --- |
| Model | CBN-E306 |
| Country of Origin | China |
| Displacement | 6 cc/rev |
| Max Pressure | 160-200 bar |
| Max Speed | 2500 rpm max |
| Construction Material | Aluminum alloy body |
| Pump Price (Est.) | USD 20-60 (Amazon/eBay) CNY 80-250 (domestic China) |
| Price Notes | Budget option, 5-15x cheaper |
| Geographic Availability | China (manufacturer — Hangzhou, Zhejiang), USA (Amazon/eBay), worldwide via AliExpress/Alibaba |

Source 1: Amazon (USA)

Source 2: Made-in-China

Analysis: Budget Chinese pump. CBN-E series widely used on Jinma, Foton, Dongfeng tractors. Aluminum construction. Flat key or spline shaft. Lowest pressure rating. No FKM seals standard (NBR only — max ~80°C). No integrated safety valve. Suitable only for low-duty applications.

## 10. Settima GR20 SMT8B (6.4cc) (Italy)

| Manufacturer | Settima |
| --- | --- |
| Model | GR20 SMT8B (6.4cc) |
| Country of Origin | Italy |
| Displacement | 6.4 cc/rev |
| Max Pressure | 25-40 bar (low pressure screw) |
| Max Speed | 1500-3000 rpm |
| Construction Material | Cast iron/aluminum |
| Pump Price (Est.) | EUR 200-500 (estimated) Higher for original Italian units |
| Price Notes | Premium silent pump technology |
| Geographic Availability | Italy (manufacturer), Germany (Settima.de distributor), South Africa (Hytec/Rexroth), China (aftermarket), worldwide |

Source 1: Settima Official (Italy)

Source 2: eBay (used/surplus)

Analysis: Italian screw pump (NOT gear pump — different technology). Ultra-low noise operation. Designed for gearbox lubrication and lube services. 6.4cc displacement close match. Low pressure only (25-40 bar) vs Scherzinger's 8-15 bar — compatible for lubrication duty. Premium silent technology.

## 11. Vivoil XP101 (Group 1, ~6cc) (Italy)

| Manufacturer | Vivoil |
| --- | --- |
| Model | XP101 (Group 1, ~6cc) |
| Country of Origin | Italy |
| Displacement | ~6 cc/rev |
| Max Pressure | 250 bar |
| Max Speed | 3000-4000 rpm |
| Construction Material | Aluminum body |
| Pump Price (Est.) | EUR 80-200 (EU distributors est.) |
| Price Notes | Good Italian value option |
| Geographic Availability | Italy (manufacturer — Budrio), UK (Flowtechnik, Hydraulics Online), Germany (Maschinenteil24), Europe-wide |

Source 1: Vivoil Official (Italy)

Source 2: UK Flowtechnik

Analysis: Italian manufacturer from Budrio (Bologna). Group 1 pumps with 25.4mm flange. Unidirectional design. Aluminum construction. Good quality-to-price ratio for European manufacturing. Available in multiple configurations through EU distributors.

## 12. Indian OEM / Rotofluid FTMP-6 / Generic 6cc (India)

| Manufacturer | Indian OEM / Rotofluid |
| --- | --- |
| Model | FTMP-6 / Generic 6cc |
| Country of Origin | India |
| Displacement | 6 cc/rev |
| Max Pressure | 100-150 bar |
| Max Speed | 1500-2000 rpm |
| Construction Material | Cast iron |
| Pump Price (Est.) | INR 3,000-8,000 (USD 36-95) (IndiaMART listings) |
| Price Notes | Lowest cost cast iron option |
| Geographic Availability | India (manufacturer — various cities), limited export |

Source 1: IndiaMART - Lubrication Pumps

Source 2: Rotofluid Pumps (India)

Analysis: Indian domestic manufacturers. FTMP series specifically designed for lubrication oil transfer. Cast iron construction matches Scherzinger material. Lowest cost globally but also lowest performance specs. Limited to Indian domestic market. Suitable for basic lubrication duty only.

# 4. Price Comparison by Country / Region

Small-displacement flange gear pumps show significant price variation by region, driven by local manufacturing, import duties, and brand positioning.

| Region | Budget | Mid-Range | Premium | Key Brands |
| --- | --- | --- | --- | --- |
| China (domestic) | CNY 80-250 (USD 11-35) | CNY 500-1,500 (USD 70-210) | CNY 2,000-5,000 (USD 275-690) | CBN, CBHS |
| India | INR 3,000-8,000 (USD 36-95) | INR 10,000-20,000 (USD 120-240) | INR 30,000-50,000 (USD 360-600) | Rotofluid, Supremo |
| USA / N. America | USD 20-60 (Chinese imports) | USD 196-350 (Marzocchi, Casappa, Honor) | USD 350-650 (Parker, Rexroth) | Parker, Chief, Honor |
| Europe (EU / UK) | EUR 80-200 (Vivoil, Danfoss) | EUR 180-400 (Casappa, Bucher, Marzocchi) | EUR 400-650 (Rexroth, KRACHT) | Rexroth, KRACHT, Bucher |
| Japan | JPY 10,000-20,000 (USD 67-135) | JPY 25,000-50,000 (USD 170-340) | JPY 50,000-100,000 (USD 340-675) | Shimadzu, Nachi |
| Middle East / Africa | USD 30-80 (Chinese imports) | USD 150-300 (Italian/Turkish) | USD 400-700 (European brands) | Mixed suppliers |

Note: All prices are pump-only. The Scherzinger 201267-A includes an integrated safety valve, which would add EUR 30-100 to competitors requiring external valves. FKM seal upgrades on competitors add EUR 20-50. These hidden costs narrow the apparent price gap significantly.

# 5. Key Findings and Competitive Position

## 5.1 Unique Selling Proposition

The Scherzinger 201267-A occupies a unique position in the market. No single competitor offers the full combination of: (1) cast iron housing, (2) integrated safety valve, (3) FKM seals rated to 160°C, (4) precision flange mount with centering ring, and (5) ground gears. The closest competitor in material quality is KRACHT KF6 (Germany), but it lacks the integrated safety valve and has a lower temperature ceiling (~150°C). Bosch Rexroth AZPF offers FKM seals as an option but at significant cost premium and without the safety valve.

## 5.2 Price Positioning

Based on the competitive landscape, the Scherzinger 201267-A likely competes in the EUR 300-600 range for the pump head alone. This is justified by the integrated safety valve (saves EUR 30-100 vs. external), FKM seals included (saves EUR 20-50 vs. upgrade), cast iron construction (vs. aluminum on most competitors), and the 160°C temperature rating that most competitors simply cannot match. When total system cost is considered (pump + external valve + seal upgrade), the Scherzinger is very competitive against Bosch Rexroth and KRACHT while offering superior temperature performance.

## 5.3 Competitive Threats

For standard-temperature applications (below 80°C), Chinese CBN-E306 pumps at USD 20-60 represent a significant price threat for cost-sensitive customers. Italian manufacturers (Marzocchi, Casappa, Vivoil, Danfoss/Turolla) offer strong mid-range alternatives in aluminum. However, for high-temperature lubrication applications requiring cast iron and FKM — Scherzinger's target market — only KRACHT and Bosch Rexroth (with options) can compete, both at similar or higher price points.

## 5.4 Application-Specific Advantages

The 201267-A is particularly well-suited for: industrial gearbox lubrication (high temp oil), power plant auxiliary systems, metalworking machine tool lubrication, chemical process support systems, and any application requiring reliable low-pressure fluid transfer at elevated temperatures. The integrated safety valve makes it especially attractive for OEM integration where space and simplicity are valued.

# 6. Complete Source Directory

All sources used in this analysis, organized by competitor. Click any link to verify pricing and specifications.

1. Marzocchi Pompe ALP1A-D-6

- OneHydraulics (USA) — https://www.onehydraulics.com/products/alp1a-d-6

- USC Hydraulics (USA) — https://uschydraulics.com/products/alp1-series-marzocchi

2. Casappa PLP10.5D0

- OneHydraulics (USA) — https://www.onehydraulics.com/products/plp10-5d0-36r9-lob-oa-n-l-casappa-polaris-gear-pump-5-34cc-3625psi-rated-4000rpm-cw-1-2-bore-x-1-8-key-shaft-sae-aa-2-4-bolt-flange-0-5-1-2-8-sae-inlet-0-375-3-8-6-sae-outlet-aluminum-body-cast-iron-flange-and-cover

- Hidraoil (Spain) — https://www.hidraoil.com/products/standard/plp20.20d0-82e2-leb-ea-n-el-fs-02004648-casappa-hydraulic-gear-pump-20-cm3-tapered-shaft-cw-rotation/

3. Bosch Rexroth AZPF-11-005 / AZPB-22-005

- Hydraulics Online (UK) — https://hydraulicsonline.com/product/bosch-rexroth-azpf-series-external-gear-pump/

- IndiaMART (India) — https://www.indiamart.com/proddetail/bosch-rexroth-azpb-azpf-azpn-azpj-external-gear-pumps-8490179330.html

4. Parker Hannifin PGP505A-0060

- Parker Official (USA) — https://ph.parker.com/us/en/product-list/high-pressure-aluminum-pumps-model-505

- GS Global Resources — https://gsglobalresources.com/product/hydraulic/parker-pgp505-series/

5. KRACHT GmbH KF6RF1-D15

- KRACHT Media (catalog) — https://www.kracht-media.eu/wp-content/uploads/KF3-KF6-EN-07-2024.pdf

- KRACHT Corp (USA) — https://www.krachtcorp.com/pump-technology/transfer-pumps/kf/

6. Danfoss (Turolla) SNP1/6.0D (or SKP1/6.0D)

- Hidraoil (Spain) — https://www.hidraoil.com/products/standard/skp1-6.0d-ci06-sauer-danfoss-turolla-hydraulic-gear-pump/

- Hydraulics Online (UK) — https://hydraulicsonline.com/product/danfoss-snp1nn-group-1-gear-pumps/

7. Bucher Hydraulics AP100/5.0D880

- Bucher Official — https://www.bucherhydraulics.com/en/products/pumps-and-motors/pumps/external-gear-pumps-ap

- Flowfit (UK) — https://www.flowfitonline.com/products/bucher-ap100-1-2s309-up100-gear-pump-1-2cc-rev-1-72l-min

8. Honor (Taiwan) 2GG1U06R

- OneHydraulics (USA) — https://www.onehydraulics.com/products/2gg1u06l

- OneHydraulics - Honor Collection — https://www.onehydraulics.com/collections/honor

9. CBN/Chinese Generic CBN-E306

- Amazon (USA) — https://www.amazon.com/YFCDZVBH-Hydraulic-CBN-E306-CBN-E314-CBN-E308/dp/B09VX2ZXBR

- Made-in-China — https://www.made-in-china.com/showroom/bristle-brush-991604/product-detailloUQYSWAXwpZ/China-Jinma-Tractor-Hydraulic-Gear-Pump-CBN-E306-CBN-E310-CBN-E314.html

10. Settima GR20 SMT8B (6.4cc)

- Settima Official (Italy) — https://www.settima.it/products/screw-pumps/smt-8b.html

- eBay (used/surplus) — https://www.ebay.com/itm/335763953323

11. Vivoil XP101 (Group 1, ~6cc)

- Vivoil Official (Italy) — https://www.vivoil.com/product/xp101/

- UK Flowtechnik — https://www.ukflowtechnik.com/group-1-hydraulic-gear-pumps

12. Indian OEM / Rotofluid FTMP-6 / Generic 6cc

- IndiaMART - Lubrication Pumps — https://dir.indiamart.com/impcat/lubrication-pumps.html

- Rotofluid Pumps (India) — https://www.rotofluidpumps.com/ftmp.htm